Edge Computing Laboratory

Lab Assignment 8

Name: Gaurav Gadekar

Class: TY AIEC Batch C

Enrollment No: MITU23BTCSD120

Roll No: D2233120

### Title  The "magic wand" project that can recognize gestures using an accelerometer and an ML classification model on Edge Devices

Objective: Build a project to detect the accelerometer values and convert them into gestures  Tasks:

- Generate the dataset for Accelerometer Motion (Up-Down, Left-Right)

- Configure BLE Sense / Mobile for Edge Impulse

- Building and Training a Model

- Deploy on Nano BLE Sense / Mobile Phone

### Introduction

Edge Impulse is a development platform for machine learning on edge devices, targeted at developers who want to create intelligent device solutions. The " Accelerometer Motion "sensor reading equivalent in Edge Impulse would typically involve creating a simple machine learning model that can run on an edge device, like classifying sensor data or recognizing a basic pattern.

Materials Required

• 	Nano BLE Sense Board

### Theory

GPIO (General Purpose Input/Output) pins on the Raspberry Pi are used for interfacing with other electronic components. BCM numbering refers to the pin numbers in the Broadcom SOC channel, which is a more consistent way to refer to the GPIO pins across different versions of the

Here’s a high-level overview of steps you'd follow to create a "Hello World" project on Edge Impulse:

Steps to Configure the Edge Impulse:

Create an Account and New Project:

  - Sign up for an Edge Impulse account.

  - Create a new project from the dashboard.

Connect a Device:

  - You can use a supported development board or your smartphone as a sensor device.

  - Follow the instructions to connect your device to your Edge Impulse project.

Collect Data:

Use the Edge Impulse mobile app or the Web interface to collect data from the onboard sensors.

  - For a "Hello World" project, you could collect accelerometer data, for instance.

Create an Impulse:

  - Go to the 'Create impulse' page.

  - Add a processing block (e.g., time-series data) and a learning block (e.g., classification).

  - Save the impulse, which defines the machine learning pipeline.

Design a Neural Network:

  - Navigate to the 'NN Classifier' under the 'Learning blocks'.

  - Design a simple neural network. Edge Impulse provides a default architecture that works well for most basic tasks.

Train the Model:

  - Click on the 'Start training' button to train your machine learning model with the collected data.

Test the Model:

  - Once the model is trained, you can test its performance with new data in the 'Model Testing' tab.

Deploy the Model:

  - Go to the 'Deployment' tab.

  - Select the deployment method that suits your edge device (e.g., Arduino library, WebAssembly, container, etc.).

  - Follow the instructions to deploy the model to your device.

Run Inference:

  - With the model deployed, run inference on the edge device to see it classifying data in real-time.

Monitor:

  - You can monitor the performance of your device through the Edge Impulse studio

Conclusion: Understood Accelerometer Motion "sensor reading equivalent in Edge Impulse would typically involve creating a simple machine learning model with low computing speed and power.

Screenshots:

- Dataset Image

- Feature extraction - Image

- Accuracy / Loss - Confusion Matrix – image

# Validation Result – Image

# Copy the code of Arduino Sketch

/* Edge Impulse ingestion SDK

* Copyright (c) 2022 EdgeImpulse Inc.

*

* Licensed under the Apache License, Version 2.0 (the "License");

* you may not use this file except in compliance with the License.

* You may obtain a copy of the License at

* http://www.apache.org/licenses/LICENSE-2.0

*

* Unless required by applicable law or agreed to in writing, software

* distributed under the License is distributed on an "AS IS" BASIS,

* WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

* See the License for the specific language governing permissions and

* limitations under the License.

*

*/

/* Includes ---------------------------------------------------------------- */

#include <Connect_Nano_BLE_inferencing.h>

#include <Arduino_LSM9DS1.h> //Click here to get the library: https://www.arduino.cc/reference/en/libraries/arduino_lsm9ds1/

/* Constant defines -------------------------------------------------------- */

#define CONVERT_G_TO_MS2    9.80665f

/**

* When data is collected by the Edge Impulse Arduino Nano 33 BLE Sense

* firmware, it is limited to a 2G range. If the model was created with a

* different sample range, modify this constant to match the input values.

* See https://github.com/edgeimpulse/firmware-arduino-nano-33-ble-sense/blob/master/src/sensors/ei_lsm9ds1.cpp

* for more information.

*/

#define MAX_ACCEPTED_RANGE  2.0f

/*

** NOTE: If you run into TFLite arena allocation issue.

**

** This may be due to may dynamic memory fragmentation.

** Try defining "-DEI_CLASSIFIER_ALLOCATION_STATIC" in boards.local.txt (create

** if it doesn't exist) and copy this file to

** `<ARDUINO_CORE_INSTALL_PATH>/arduino/hardware/<mbed_core>/<core_version>/`.

**

** See

** (https://support.arduino.cc/hc/en-us/articles/360012076960-Where-are-the-installed-cores-located-)

** to find where Arduino installs cores on your machine.

**

** If the problem persists then there's not enough memory for this model and application.

*/

/* Private variables ------------------------------------------------------- */

static bool debug_nn = false; // Set this to true to see e.g. features generated from the raw signal

static uint32_t run_inference_every_ms = 200;

static rtos::Thread inference_thread(osPriorityLow);

static float buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE] = { 0 };

static float inference_buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE];

/* Forward declaration */

void run_inference_background();

/**

* @brief      Arduino setup function

*/

void setup()

{

// Initialize pins as outputs

pinMode(LEDR, OUTPUT);

pinMode(LEDG, OUTPUT);

pinMode(LEDB, OUTPUT);

// put your setup code here, to run once:

Serial.begin(115200);

// comment out the below line to cancel the wait for USB connection (needed for native USB)

while (!Serial);

Serial.println("Edge Impulse Inferencing Demo");

if (!IMU.begin()) {

ei_printf("Failed to initialize IMU!\r\n");

}

else {

ei_printf("IMU initialized\r\n");

}

if (EI_CLASSIFIER_RAW_SAMPLES_PER_FRAME != 3) {

ei_printf("ERR: EI_CLASSIFIER_RAW_SAMPLES_PER_FRAME should be equal to 3 (the 3 sensor axes)\n");

return;

}

inference_thread.start(mbed::callback(&run_inference_background));

}

/**

* @brief Return the sign of the number

*

* @param number

* @return int 1 if positive (or 0) -1 if negative

*/

float ei_get_sign(float number) {

return (number >= 0.0) ? 1.0 : -1.0;

}

/**

* @brief      Run inferencing in the background.

*/

void run_inference_background()

{

// wait until we have a full buffer

delay((EI_CLASSIFIER_INTERVAL_MS * EI_CLASSIFIER_RAW_SAMPLE_COUNT) + 100);

// This is a structure that smoothens the output result

// With the default settings 70% of readings should be the same before classifying.

ei_classifier_smooth_t smooth;

ei_classifier_smooth_init(&smooth, 10 /* no. of readings */, 7 /* min. readings the same */, 0.8 /* min. confidence */, 0.3 /* max anomaly */);

while (1) {

// copy the buffer

memcpy(inference_buffer, buffer, EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE * sizeof(float));

// Turn the raw buffer in a signal which we can the classify

signal_t signal;

int err = numpy::signal_from_buffer(inference_buffer, EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE, &signal);

if (err != 0) {

ei_printf("Failed to create signal from buffer (%d)\n", err);

return;

}

// Run the classifier

ei_impulse_result_t result = { 0 };

err = run_classifier(&signal, &result, debug_nn);

if (err != EI_IMPULSE_OK) {

ei_printf("ERR: Failed to run classifier (%d)\n", err);

return;

}

// print the predictions

ei_printf("Predictions ");

ei_printf("(DSP: %d ms., Classification: %d ms., Anomaly: %d ms.)",

result.timing.dsp, result.timing.classification, result.timing.anomaly);

ei_printf(": ");

// ei_classifier_smooth_update yields the predicted label

const char *prediction = ei_classifier_smooth_update(&smooth, &result);

ei_printf("%s ", prediction);

// print the cumulative results

ei_printf(" [ ");

for (size_t ix = 0; ix < smooth.count_size; ix++) {

ei_printf("%u", smooth.count[ix]);

if (ix != smooth.count_size + 1) {

ei_printf(", ");

}

else {

ei_printf(" ");

}

}

ei_printf("]\n");

delay(run_inference_every_ms);

}

ei_classifier_smooth_free(&smooth);

}

/**

* @brief      Get data and run inferencing

*

* @param[in]  debug  Get debug info if true

*/

void loop()

{

while (1) {

// Determine the next tick (and then sleep later)

uint64_t next_tick = micros() + (EI_CLASSIFIER_INTERVAL_MS * 1000);

// roll the buffer -3 points so we can overwrite the last one

numpy::roll(buffer, EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE, -3);

// read to the end of the buffer

IMU.readAcceleration(

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 3],

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 2],

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 1]

);

for (int i = 0; i < 3; i++) {

if (fabs(buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 3 + i]) > MAX_ACCEPTED_RANGE) {

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 3 + i] = ei_get_sign(buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 3 + i]) * MAX_ACCEPTED_RANGE;

}

}

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 3] *= CONVERT_G_TO_MS2;

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 2] *= CONVERT_G_TO_MS2;

buffer[EI_CLASSIFIER_DSP_INPUT_FRAME_SIZE - 1] *= CONVERT_G_TO_MS2;

// and wait for next tick

uint64_t time_to_wait = next_tick - micros();

delay((int)floor((float)time_to_wait / 1000.0f));

delayMicroseconds(time_to_wait % 1000);

char prediction;

if (prediction == "idle")

{

// WHITE

digitalWrite(LEDR, LOW);

digitalWrite(LEDG, LOW);

digitalWrite(LEDB, LOW);

}

else if (prediction == "left-right"){

// RED

digitalWrite(LEDR, LOW);

digitalWrite(LEDG, HIGH);

digitalWrite(LEDB, HIGH);

// wait for a second

delay(1000);

}

else if (prediction == "up-down"){

// GREEN

digitalWrite(LEDR, HIGH);

digitalWrite(LEDG, LOW);

digitalWrite(LEDB, HIGH);

// wait for a second

delay(1000);

}

else if (prediction == "circular"){

// BLUE

digitalWrite(LEDR, HIGH);

digitalWrite(LEDG, HIGH);

digitalWrite(LEDB, LOW);

// wait for a second

delay(1000);

}

}

}

#if !defined(EI_CLASSIFIER_SENSOR) || EI_CLASSIFIER_SENSOR != EI_CLASSIFIER_SENSOR_ACCELEROMETER

#error "Invalid model for current sensor"

#endif

# Screen shot of Arduino Terminal – Result